BP MONITORING SYSTEM Hardware & Software Requirements Specification

⚙  Hardware Requirements

| Component | Part | Description |
| --- | --- | --- |
| Microcontroller | ESP8266 (NodeMCU / Wemos D1 Mini) | 3.3V logic, built-in Wi-Fi, D1/D2. Streams BPM to Firebase. |
| Sensor | MAX30102 | Optical heart-rate sensor. Reads raw IR; BPM computed on-device and pushed to /live/bpm. |
| Status LEDs | RGB Light Module | Red = alert, Green = normal BPM, Blue = low, blinking purple = sensor not found. |
| Wiring | Jumper Wires | Female to Female: VIN, GND, SDA (D2), SCL (D1), 3 GPIO pins for LEDs and 1 GND. |
| Power | USB / 3.3V Supply | Minimum 500 mA recommended. ESP8266 draws up to ~300 mA during Wi-Fi bursts. |

💻  Software Requirements

IDE: Arduino IDE 2.3.8 Board Manager URL: http://arduino.esp8266.com/stable/package_esp8266com_index.json

| Library | Purpose |
| --- | --- |
| Wire.h | Connects ESP8266 to MAX30102 via SDA/SCL pins. |
| MAX30102 | Community sensor driver — initializes chip, reads raw Red & IR values. |
| ESP8266WiFi.h | Manages Wi-Fi connection for cloud data upload to Firebase. |
| Google Firebase | BPM is stored in Firebase Realtime Database over HTTPS. |